HamComm 3.1                  Manual               June 10. 1996

HamComm

Version 3.1

June 10. 1996

W. F. Schroeder

DL5YEC

1

HamComm 3.1                  Manual               June 10. 1996

Table of contents

1 Introduction ...............................................4

2 License ....................................................4

3 Update .....................................................4

4 Exclusion of warranty ......................................5

5 System requirements ........................................5

6 Installation ...............................................5

7 Startup ....................................................6

8 Config file HC31.CFG .......................................6

9 Screen layout ..............................................7

10 Help system ...............................................7

11 File menu .................................................7

11.1 Start RX logging ......................................7

11.2 Save RX Win & log .....................................8

11.3 Start/Stop TX logging .................................8

11.4 Log to printer ........................................8

11.5 Macro recorder ........................................8

11.6 Load macros ...........................................8

11.7 Save macros ...........................................8

11.8 Load config file ......................................9

11.9 Send text file ........................................9

11.10 About HamComm ........................................9

11.11 Shell to DOS .........................................9

11.12 Exit to DOS ..........................................9

12 Mode menu .................................................9

12.1 TX on/off .............................................9

12.2 CW ....................................................9

12.3 Baudot ................................................9

12.4 ASCII 7/8 ............................................10

12.5 AMTOR Listen .........................................10

12.6 AMTOR ARQ ............................................10

12.7 AMTOR FEC ............................................10

12.8 Pactor Listen ........................................10

12.9 Bit Length ...........................................11

12.10 Spectrum ............................................11

12.11 Scope ...............................................11

12.12 Tune ................................................11

13 Speed menu ...............................................11

13.1 Baud selection .......................................11

13.2 WPM selection ........................................11

13.3 RX AutoWPM ...........................................11

14 Keying menu ..............................................12

14.1 Normal/Reverse .......................................12

14.2 170Hz, 425Hz, 850Hz, Set var .........................12

14.3 Diddle ...............................................12

14.4 Clear TX Buffer ......................................12

14.5 External Converter ...................................12

14.6 AFC ..................................................12

15 Port menu ................................................12

15.1 COM1 - COM4 ..........................................12

15.2 Use Speaker ..........................................12

16 Text menu ................................................13

16.1 Send Text ............................................13

16.2 Define Text ..........................................13

16.3 Define Call/Name/Selcall .............................13

16.4 Set QSO count ........................................13

16.5 Send QSO count .......................................13

16.6 Advance and send QSO count ...........................13

2

HamComm 3.1                  Manual               June 10. 1996

16.7 AMTOR AAB ............................................13

16.8 AutoUnshift ..........................................13

16.9 Blank lines ..........................................13

16.10 Extended Baudot .....................................14

16.11 Word/Character mode .................................14

16.12 WX decoder ..........................................14

16.13 Force SHIP/SYNOP decode .............................15

17 Info menu ................................................15

17.1 Locator ..............................................15

17.2 Callsign .............................................15

17.3 HamComm ..............................................15

17.4 System ...............................................15

18 Options menu .............................................15

18.1 Clear RX/TX window ...................................15

18.2 Set ClockCorr ........................................15

19 Interface schematics .....................................16

20 Receive circuit ..........................................16

21 PTT circuit ..............................................16

22 Transmit circuit .........................................17

23 External converter .......................................17

24 External AFSK ............................................17

25 Signal decoding ..........................................18

26 RTTY Basics ..............................................18

27 RTTY Decoding ............................................19

28 Morse code basics ........................................19

29 Morse code decoding ......................................20

30 SITOR A/B ................................................21

31 NAVTEX ...................................................21

32 AMTOR ....................................................22

33 FEC monitoring ...........................................22

34 FEC transmission .........................................23

35 ARQ LISTEN mode ..........................................23

36 ARQ transmission .........................................23

37 Error correction .........................................24

38 Clock correction .........................................25

39 Macro keys ...............................................26

40 SHIP/SYNOP decoder .......................................26

41 Scope function ...........................................28

42 Spectrum function ........................................29

43 Bitlength statistics .....................................30

44 Interface checkout .......................................31

45 Port address .............................................31

46 Op-Amp power supply ......................................32

47 IRQ number ...............................................34

48 Appendix A - Key assignment summary ......................35

49 Appendix B - Baudot code table ...........................38

50 Appendix C - Interface schematics ........................39

51 Appendix D - Interface supplier ..........................41

52 Index ....................................................42

3

HamComm 3.1                  Manual               June 10. 1996

1 Introduction

HamComm is a program for ham radio communications.  Version 3.1

supports reception and transmission of radio teletype and Morse

code  signals.    Available  modes  are  BAUDOT,  ASCII,  AMTOR

ARQ/FEC, SITOR  A/B  and NAVTEX.    For PACTOR  transmission  a

'Listen' mode is  available.   Weather station  reports in  the

SHIP and SYNOP format can also be decoded.

A conventional converter or  modem chip is  not required.   The

audio output of the receiver is connected to the serial port of

a PC-compatible computer thru  a very simple low-cost  circuit.

Only one inexpensive IC is needed (Op-Amp TL071 or similar) and

a few  diodes,  capacitors and  resistors.   A  separate  power

supply is not required,  the supply current  is drawn from  the

serial port.    For  transmission the  audio  signal  from  the

speaker or serial port is connected to the microphone input  of

the transmitter thru  a passive  R/C filter.   Audio  frequency

generation and  decoding,  serial/parallel conversion  and  all

other signal processing is done by the program.

2 License

HamComm 3.1 is not a "public domain" program.

Copyright (c) 1990-1996 by W.F. Schroeder

All rights reserved.

HamComm  3.1 is  shareware.   The unregistered  version may  be

freely copied and distributed.   The program and/or manual  may

not be modified in any way without the prior written permission

by the author.  It is not allowed to charge more than US$10 for

distribution.

You are hereby granted a limited  license to use this  software

for evaluation purposes for a period of 30 days.  If you intend

to continue using  this software  after the  30 day  evaluation

period, you must make a registration  payment of US$30 or  DM40

to:

W. F. Schroeder

Augsburger Weg 63

D-33102 Paderborn

Germany

Please note: the bank will  charge a fee up  to DM10 to cash  a

cheque drawn on a foreign (non-german) bank.  For this reason i

can only  accept cheques  about DM40  drawn on  a german  bank.

Alternatively you can send an international postal money  order

or banknotes (DM or equivalent amount in local currency).

3 Update

For registered  users  of  previous versions,  HamComm   3.1 is

available for DM15 in Germany or US$15 to all other countries.

4

HamComm 3.1                  Manual               June 10. 1996

4 Exclusion of warranty

The author  provides  absolutely  no warranty,  to  the  extent

permitted by  applicable  state  law.   Except  when  otherwise

stated in writing, the author and/or other parties provide this

program "AS IS" without warranty of any kind, either  expressed

or  implied,  including,  but  not  limited  to,  the   implied

warranties of  merchantability  and fitness  for  a  particular

purpose.  The entire risk as to the quality and performance  of

the program is with you.   Should the program prove  defective,

you assume  the  cost of  all  necessary servicing,  repair  or

correction.

In no event unless required by  applicable law will the  author

and/or any other party who may modify and/or redistribute  this

program be  liable  to  you for  damages,  including  any  lost

profits,  lost  monies,   or  other   special,  incidental   or

consequential damages arising  out of the  use or inability  to

use (including but not  limited to loss of  data or data  being

rendered inaccurate or losses sustained  by third parties or  a

failure of the program to operate with any other programs) this

program, even if you  have been advised  of the possibility  of

such damages, or for any claim by any other party.

5 System requirements

HamComm  3.1 will run  under MS-DOS  3.x or  higher on  any PC-

compatible computer  with at  least 400KB  of free  memory.   A

harddisk is highly  recommended.  Because  of the  size of  the

program file there may be not enough space to copy all required

files onto a 720KB diskette.   On slow systems, e.g. 8088  CPU,

some functions may not work as expected or may not work at all.

HamComm will automatically detect the type of video adapter  in

use.   MDA, CGA,  EGA, VGA  and Hercules  are supported.    The

graphics display functions are not available on MDAs since  the

MDA has no graphics  mode.  No attempt  has been made to  avoid

screen flicker (snow) on cheap CGAs.

HamComm will probably  not run under  any kind of  multitasking

software like  Desqview, Windows  3.x/95/NT  or OS/2  since  it

requires direct control of the interrupt controller, timer chip

and serial I/O hardware.

6 Installation

For installation on a harddisk you should create a subdirectory

`HAMCOMM' and change to that directory.  HamComm is distributed

as a .ZIP  or .EXE  file.  To  unpack a  .ZIP file  you need  a

program like UNZIP or  PKUNZIP.  The .EXE  version is a  'self-

extracting archive' which contains all the files and a build-in

unpack program.   In  both cases  the extracted  files will  be

placed in the current directory.

Please read the files README, HC31.CFG and CHANGES !

5

HamComm 3.1                  Manual               June 10. 1996

7 Startup

To start HamComm type HC at the DOS prompt and hit ENTER.   The

interface circuit is not needed if  you just want to play  with

the program.

HamComm  3.1 normally  uses  the video  mode  with the  maximum

number of lines, e.g. 50 lines on  VGA, 43 lines on EGA and  25

lines with other video cards.  If  you want to use a mode  with

less line, specify option ` -L43' to switch to 43 lines  on VGA

cards or `-L25' for 25 lines on EGA/VGA cards.

Many SuperVGA  cards have  special text  modes e.g.   80x60  or

132x44.   Activate the  desired mode  before starting  HamComm.

Most cards come with  a utility program to  do this.  Only  the

modes where the video display buffer starts at segment B800 are

supported.  Start HamComm with the commandline option `-LX' for

automatic  determination  of  the  number  of  text-lines   and

columns.  Some  mouse drivers have  problems with the  extended

text modes,  as they  are not  smart  enough to  recognize  the

screen dimensions.

HamComm assumes  that the  original  PC display  character  set

known as "codepage 437" is active.

On startup the program identifies the video card and monitor in

use and  automatically selects  the graphics  mode for  highest

resolution.  Some program functions require a graphics mode for

display of the input signal.

The identification may fail because of compatibility  problems.

By selecting one of  the following commandline options  HamComm

can be forced to use the specified mode:

Option     Mode        Resolution

-cga       6           640x200  2-color

-ega       10h         640x350 16-color

-vga       12h         640x480 16-color

-herc      Hercules    720x348  2-color

If you have a computer with  an LCD display and readability  is

poor try the following DOS command before starting HamComm:

mode bw80

The 'mode' program is supplied with MS-DOS and is used here  to

switch to black and white mode.

8 Config file HC31.CFG

When HamComm is started the program automatically searches  for

a configuration file called "HC31.CFG".  The current  directory

is searched first, then all other directories along the current

"PATH".  The supplied HC31.CFG file  is an ASCII text file  and

can be sent to a printer or changed with a text editor.  Please

take the  time  to read  it  carefully and  make  the  required

6

HamComm 3.1                  Manual               June 10. 1996

adjustments for your  system.  At  least you should  substitute

DL5YEC with your own callsign.

HC31.CFG includes explanations and  examples for all  available

configuration commands.   It  is only  special  in that  it  is

executed automatically on program startup.  You can also  write

your own .CFG files and execute  them from the FILE menu  while

running HamComm.  This could be used to set several  parameters

like Baud and shift to special  values in one step.  You  could

also build a set  of .CFG files for  loading standard texts  in

different languages.  In combination  with the macro keys  they

could then be loaded with a single key.

9 Screen layout

HamComm has an  SAA-like user interface  with pull-down  menus,

dialog boxes and online helptexts.  The menu bar at the top  of

the screen is always visible while the display is in text mode.

The bottom line  has information about  the currently  selected

menu item.

Every name on the menu bar has a highlighted character.   Press

and hold the  ALT key  and type  that character  to select  the

corresponding menu.  The cursor-left and cursor-right keys  can

now be  used to  switch to  the previous/next  menu.   Use  the

cursor-up and cursor-down keys to move to the desired  function

and hit the  ENTER key.   Selecting a mode  from the mode  menu

will get you to the main transmit/receive (RX/TX) screen.

Every menu line also has a highlighted character.  Typing  that

character (without the ALT  key) will get  you directly to  the

corresponding function.

Some  frequently  used  menu  entries  have  been  assigned  to

function keys.   The  F8 key  for example  activates the  SCOPE

function.

The ESC  key  can  be  used  at any  time  to  abort  the  menu

selection.

10 Help system

The ALT-H  and F1  keys both  start the  help system  but  show

different kinds of information.  The ALT-H key displays general

information about the program, the F1 key displays a help  text

specific to the current situation.

11 File menu

11.1 Start RX logging

The text appearing  in the receive  window can be  logged to  a

file.  You will be asked to enter the name of the logfile  with

a suggested file type of  ``.LOG``.  If the  logfile is active,

its name is displayed on the upper left border of the  transmit

window.  The text of the entry  in the FILE menu is changed  to

7

HamComm 3.1                  Manual               June 10. 1996

``Stop RX logging`` and it will close the  logfile if selected.

The  file  will  also   be  closed  automatically  on   program

termination.

Whenever a new  line begins in  the logfile,  the text  element

`logrxlabel'  is  inserted  in   receive  mode  and  the   text

`logtxlabel' is inserted  in transmit mode.   See the  supplied

configuration file   HC31.CFG for  a  sample  definition  which

inserts the current time.

This menu entry is only available after selecting an  operating

mode from the MODE menu.

11.2 Save RX Win & log

This entry is almost identical to `Start RX logging'  described

above.  The  contents of  the RX window  is saved  to the  file

before the normal logging operation begins.

This menu entry is only available after selecting an  operating

mode from the MODE menu.

11.3 Start/Stop TX logging

Open/Close the logfile for the TX window.

11.4 Log to printer

All text  appearing in  the RX  window will  be copied  to  the

device PRN which is normally connected to LPT1.  Make sure that

the printer is ready to print  before activation.  A  checkmark

is displayed in front of the menu entry while logging.

11.5 Macro recorder

This menu  entry is  used to  start the  recoder for  keystroke

sequences which can be assigned to and replayed with the  macro

keys.   Please  refer to  section  `Macro keys'  (p. 26) for  a

detailed discussion

11.6 Load macros

To load  a previously  saved  ``.MAC``macro  file select  "Load

macros" from the FILE menu.  All macro definitions found in the

selected  file  will  be  added  to  or  replace  the   current

assignments.  Macro keys that didn't hold any keystrokes at the

time the file was written, will not be changed when the file is

loaded.

There is also a way to  load your favourite macros  definitions

automatically.  When  HamComm is  started it  searches for  the

default macro file HC.MAC, first in the current directory, then

in every directory along the PATH.

11.7 Save macros

Macro definitions can  be saved to  a file  by selecting  "Save

macros" from the FILE menu.  The usual file select box pops  up

with a  suggested   .MAC file  type.    All recorded  keystroke

sequences will be saved to the selected file.

8

HamComm 3.1                  Manual               June 10. 1996

11.8 Load config file

This menu entry  is used to  select and  process config  files.

The file select box shows the  .CFG files found in the  current

directory.  Enter the name of the file to be processed or  pick

one from the list.

11.9 Send text file

The text  from the  selected file  will be  transferred to  the

transmit buffer.

This menu entry is only available after selecting an  operating

mode from the MODE menu.

11.10 About HamComm

This command displays the program date and version.

11.11 Shell to DOS

This menu entry starts a copy  of the DOS command  interpreter.

HamComm remains in memory and the receive and transmit routines

continue to execute  in the background.   The available  memory

will usually be  very limited, so  only small  programs can  be

run.

11.12 Exit to DOS

Terminates the program and returns to DOS.  Transmit mode  will

be terminated and any open files will be closed.

12 Mode menu

12.1 TX on/off

This command switches  from receive to  transmit mode and  vice

versa.   The same  function is  assigned to  the Control-T  key

while the RX/TX screen is active.

12.2 CW

Activates  the  RX/TX  screen  and  selects  the  CW  mode  for

reception and transmission of Morse code.   See the SPEED  menu

for automatic  or manual  speed adjustment.   The  current  WPM

(Words Per  Minute)  setting  is shown  at  the  bottom  window

border.  For more  information see the  sections on Morse  code

basics (p.19) and decoding.

12.3 Baudot

Activates the RX/TX screen and  selects the 5-bit Baudot  code.

Select other  required parameters  from  the SPEED  and  KEYING

menus.  For more  information see the  sections on RTTY  basics

(p.18) and decoding.

9

HamComm 3.1                  Manual               June 10. 1996

12.4 ASCII 7/8

These commands are  similar to the  Baudot selection above  but

use the 7-bit ASCII code or the PCs extended 8-bit code.  ASCII

mode is rarely  used and  was only  included for  completeness.

Tuning, speed selection and keying direction are done just like

with the Baudot mode.

12.5 AMTOR Listen

AMTOR  Listen   mode  is   used   for  monitoring   AMTOR   ARQ

transmissions from  ham radio  stations and  SITOR A  used  for

maritime communications.  See  the sections 'AMTOR' (p. 22) and

'ARQ LISTEN mode' (p.23) below for more information.

12.6 AMTOR ARQ

Select this menu entry  for two-way communication with  another

AMTOR station.   The  program checks  the received  signal  and

automatically responds to incoming ARQ  calls.  To initiate  an

AMTOR link enter the SELCALL of the other station from the TEXT

menu and then  switch to  TX mode.   See  the sections  'AMTOR'

(p.22)  and   'ARQ   transmission'  (p. 23)  below   for   more

information.

12.7 AMTOR FEC

AMTOR FEC mode is mainly used by radio amateurs for calling  CQ

(call to all stations), but also by coast stations for  weather

broadcasts and traffic lists.  The speed is always 100 Baud and

is set internally.  Set the keying direction to `normal' on USB

and `reverse' on LSB.  For more information see the sections on

`FEC monitoring' (p.22) and `FEC transmission' (p.23) below.

12.8 Pactor Listen

This mode is  used for  monitoring Pactor  transmissions.   The

speed is normally 200 Baud but drops back to 100 Baud under bad

conditions.  The  speed and  the keying  direction is  detected

automatically, so  you only  have to  tune in  to the  station.

Pactor FEC, which is  used for calling CQ,  has the same  frame

structure as the Pactor ARQ mode and will also be decoded.

Pactor uses a  CRC checksum for  error detection and  sometimes

Huffman compression for higher thruput.  Unlike with AMTOR  ARQ

damaged frames can not be decoded in part, so you either get  a

perfect copy or miss the frame completely.  If HamComm  detects

a missing frame it displays a dot and starts a new line.

The status  field  in  the lower  left  corner  has  a  quality

indicator (0 - 4 for 200 Baud, 0 - 9 for 100 Baud), the  letter

`H' to indicate  Huffman encoding, the  speed (100/200) of  the

last decoded frame and the status byte from the last frame with

the frame number.

10

HamComm 3.1                  Manual               June 10. 1996

12.9 Bit Length

Changes to  the  graphics screen  for  `Bitlength  statistics'.

This display  is  used  to  check  the  speed  of  the  decoded

mark/space signal for Baudot and ASCII transmissions.  See  the

section `Bitlength statistics' (p.30) for more information.

12.10 Spectrum

Changes to the `Spectrum'  screen.  This  display has the  best

frequency resolution  and can  therefore  be used  for  precise

tuning and to measure  the shift of the  received signal.   See

the section `Spectrum function' (p.29) for more information.

12.11 Scope

Changes to the `Scope' screen, a graphics display of the signal

frequency versus time.  See the section `Scope function' (p.28)

for more information.

12.12 Tune

The `Tune' screen is used to  set the center frequency for  the

mark and  space tones.   It  also shows  the frequency  of  the

incoming signal.  To set the  center frequency use the  cursor-

left/right keys, or click on  the highlighted fields below  the

scale.    You  can  also  click  on  the  lower  scale.     The

LOWER/HIGHER  fields  move   the  center  frequency   indicator

left/right.  Click on the RESET field or hit the `Home' key  to

restore the last  value set from  a .CFG file.   There are  two

more fields for predefined values of 1360Hz and 2210Hz.

13 Speed menu

13.1 Baud selection

The first part  of the  SPEED menu  selects the  speed for  the

asynchronous modes Baudot  and ASCII.   Pick a  speed from  the

predefined values  or enter  it directly  using the  `Set  var'

entry.  The speed for the synchronous modes AMTOR and Pactor is

part of the transmission protocol and can not be changed.

13.2 WPM selection

The second part of the menu selects the speed in WPM (Words Per

Minute) for the CW mode.

13.3 RX AutoWPM

Toggles the automatic  CW speed tracking  while receiving.   If

this option is  enabled HamComm automatically  adjusts the  WPM

setting according to  the received  signal.   For manual  speed

selection disable this option and select the desired WPM value.

11

HamComm 3.1                  Manual               June 10. 1996

14 Keying menu

14.1 Normal/Reverse

These menu  items  select the  keying  direction both  for  the

received and transmitted signals.  With direction `normal'  the

higher tone indicates the making (idle) state.

14.2 170Hz, 425Hz, 850Hz, Set var

These  menu  items  select  the  shift  for  the  received  and

transmitted signals.   The  shift is  the frequency  difference

between the mark and space tones.  Select one of the predefined

values or enter the desired value using the `Set var' entry.

14.3 Diddle

If HamComm runs out of text  while in Baudot or ASCII  transmit

mode  the  `Diddle'  option   controls  transmission  of   idle

characters.  If this option is disabled a constant `mark'  tone

is generated.

14.4 Clear TX Buffer

Clears the buffer for outgoing text.

14.5 External Converter

If this  option is  enabled, HamComm  will use  the  mark/space

signal from an external converter for decoding.  The signal  is

expected at the CTS pin of the selected serial port.

14.6 AFC

AFC stands for Automatic Frequency Control.  If this option  is

activated the center frequency  will be constantly adjusted  to

track the received signal.  It  can be used for automatic  fine

tuning or compensation  of receiver  drift.   The AFC  function

should only be used for modes with a constant tone like Baudot,

ASCII and AMTOR/Pactor FEC.

15 Port menu

15.1 COM1 - COM4

Selects and initialises the serial port  to be used.  See  also

the `select port ...' command in HC31.CFG.

15.2 Use Speaker

HamComm normally  generates the  transmit tones  using the  TxD

line of the serial port.  If this option is activated the tones

are also send to the speaker.

12

HamComm 3.1                  Manual               June 10. 1996

16 Text menu

16.1 Send Text

Displays a  dialog box  containing all  currently defined  text

elements.  Select the text to be transmitted and hit the  ENTER

key or double-click  on the  desired item.   The  text will  be

transferred to the TX window.  Hit ESC to terminate the  dialog

box.

16.2 Define Text

Defines a text element  `on the fly'.   Enter the text-ID  (the

name of the text) and the  text string in the displayed  dialog

box.  Please note that the text will not be saved to disk.   To

define a permanent text see the examples in file HC31.CFG.

16.3 Define Call/Name/Selcall

These menu items also display the `Define Text' dialog box, but

the text-ID field is preset to `call', `name' or `selcall'.

16.4 Set QSO count

Displays a dialog box  where you can set  the value of the  QSO

counter used from contest operation.

16.5 Send QSO count

Transfers the current QSO count to the TX window.

16.6 Advance and send QSO count

Increments the QSO counter  by one and  transfers its value  to

the TX window.

16.7 AMTOR AAB

Enable/disable Automatic-Answer-Back for AMTOR ARQ links.  If a

WRU (Who aRe yoU?)  character is received  during an AMTOR  ARQ

link and  this  option is  enabled  and the  text  AmtorAAB  is

defined, the  text will  automatically  be transmitted.    This

feature is normally used to  automate the login procedure  when

connecting to an AMTOR mailbox.

16.8 AutoUnshift

In Auto-Unshift  mode HamComm  will automatically  switch  from

figures to  letters in  BAUDOT mode  on  reception of  a  space

character.

16.9 Blank lines

HamComm normally ignores blank lines  to conserve space in  the

receive window.  If this option is enabled blank lines will  be

displayed.

13

HamComm 3.1                  Manual               June 10. 1996

16.10 Extended Baudot

HamComm will use the extended Baudot code for AMTOR and RTTY if

this option is enabled.  Extended Baudot was invented by  Peter

Martinez, G3PLX, to transfer the full printable ASCII character

set over AMTOR links.

16.11 Word/Character mode

These options select  the entry mode  for the TX  window.   The

current entry mode can  be LINE, WORD  or CHAR (character)  and

controls the text transfer from the TX window to the TX buffer.

The LINE mode is active if  neither WORD mode nor CHAR mode  is

seleted.

LINE mode:

1.  When a  character is typed in  the last position of  a

line, the last word  on the line will  wrap around to  the

next line.   The  current  line will  be  send to  the  TX

buffer,   followed    by    a    Carriage-Return/Line-Feed

combination.

2.  When the 'Enter' key is hit, the current line will  be

send to the TX buffer, followed by a Carriage-Return/Line-

Feed combination.

WORD mode:

1.   The spacebar,  '?' and  the  EOT character  are  word

terminators.  When a terminator is entered, all characters

between the cursor and the first terminator to the left of

the cursor  (or  the left  window  border) will  be  send.

Thereafter the terminating character  will be send to  the

TX buffer and the TX window.

2.  When the 'Enter' key  is hit, all characters from  the

first terminator to the  left of the  cursor (or the  left

window border) up to and  including the last character  on

the line will be send.  Thereafter a Carriage-Return/Line-

Feed combination will be send to the TX buffer and the  TX

window.

3.  When a  character is typed in  the last position of  a

line, the last word  on the line will  wrap around to  the

next line.   Only a Carriage-Return/Line-Feed  combination

will be send to the TX buffer.

CHAR mode:

All text will be displayed in  the TX window and put  into

the TX buffer immediately.

16.12 WX decoder

Enables/disables the decoder for weather observation data  from

SHIP and SYNOP reports.

14

HamComm 3.1                  Manual               June 10. 1996

16.13 Force SHIP/SYNOP decode

Enables the weather decoder and forces the decoder to interpret

incoming data as SHIP or SYNOP reports.

17 Info menu

17.1 Locator

Calculates the distance and direction  from one QTH locator  to

another.  A  dialog box appears  with edit fields  for the  two

locators.  The first field will be preset to the string defined

by the  text `mylocator'.   See  file HC31.CFG  for samples  of

predefined texts.

This function can also be assigned to a mouse button.  See  the

file HC31.CFG for details.

17.2 Callsign

Attempts to find the country  corresponding to a given  amateur

radio  call  sign.    You  don't  have  to  type  in  the  call

completely, the first two or three characters will usually do.

This function can also be assigned to a mouse button.  See  the

file HC31.CFG for details.

17.3 HamComm

Displays information  about  the  current  program  setup  like

buffer sizes and serial port configuration.

17.4 System

Displays information about  the system like  the type of  video

adapter and remaining free memory.

18 Options menu

18.1 Clear RX/TX window

Clears the contents of the receive or transmit window.

18.2 Set ClockCorr

Enter a clock correction value.  This setting is essential  for

reliable AMTOR operation.  For more information please refer to

section `Clock correction' (p.25).

Note that the  value you enter  in the dialog  box will not  be

saved. .  The dialog was  added to ease the process of  finding

the proper value for  your PC.  To  make the setting  permanent

you will have to change the  "set clockcorr .." command in  the

file HC31.CFG

15

HamComm 3.1                  Manual               June 10. 1996

19 Interface schematics

The interface schematics are included in the online help texts.

Start the program,

Hit ALT-H to start the help subsystem,

Hit the letter 'O' to select topic 'Overview',

Hit the TAB key to select topic 'Converter' and hit return,

Scroll the help text using the cursor-up/cursor-down keys and

read the text.

Hit ESC to leave the help system.

See also appendix C for a plain ASCII version.

20 Receive circuit

The operational amplifier  (OpAmp) is used  to bring the  audio

signal from the receiver up to RS232 level.  The supply current

is drawn from the  DTR and RTS  pins of the  serial port.   The

four diodes (1N4148 or similar) of  the receive circuit form  a

standard bridge rectifier.   The  1uF capacitors  are used  for

buffering.

The input signal  amplitude should be  at least  100mVpp.   The

100nF capacitor removes any DC bias.  Since the OpAmp runs with

maximum gain  there  will  be  a  (more  or  less)  rectangular

waveform at its  output.   It should  have an  amplitude of  at

least +/-5V to reliably drive the RS232 input.

The operational  amplifier  LM741  was choosen  because  it  is

inexpensive  and  widely  available.     However  with   todays

technology its electrical characteristics  are not  impressive.

An LF356 or TL071 for example  is much faster and has a  higher

input resistance.  If you want to substitute a different  opamp

type, keep in  mind that  the serial  port can  only deliver  a

supply current of a few milliamperes.

21 PTT circuit

The RTS output of the COM port is not only used to provide  the

supply current for the opamp, but also to key the  transmitter.

A Diode  is used  to protect  the base  of the  PTT  transistor

against the negative voltage of the RTS output in receive mode.

The resistor  is  required  to limit  the  base  current.    In

transmit mode the RTS and DTR pins change polarity.  RTS is now

positive and the transistor pulls the PTT line to ground.  This

circuit has been  tested with an  FT747, FT757GXII, TS440S  and

TS-950SD.  On some old rigs there may be a high voltage on  the

PTT line.  The required current to key the transmitter may also

be too  high for  a small  transistor, so  better check  BEFORE

connecting it to the computer.

16

HamComm 3.1                  Manual               June 10. 1996

22 Transmit circuit

AFSK tone signals for transmission are available at two places:

at the speaker connector

at the serial port  (recommended)

With many motherboards one side of the speaker is connected  to

the +5Volt supply thru a resistor, the other side is pulled  to

ground by a transistor.  In contrast to most other areas of the

PC's hardware  there  is no  standard  design for  the  speaker

output, so there may be variations.  In general one side of the

speaker will be near ground or +5Volt, while the other side has

a square-wave signal.  This signal is fed to two R/C filters to

smooth it out.   The microphone  input is very  sensitive so  a

variable resistor is  used for attenuation  and a capacitor  to

remove any DC.

If you don't want to make any modifications to the PC or if the

speaker output  is not  accessible, you  can also  get an  AFSK

signal from the TxD pin of  the COM port.  The frequencies  are

not quite as accurate as at  the speaker output and will  often

deviate by 5 to 10Hz, but this usually gives no problems.   The

signal level at the TxD pin is much higher than at the  speaker

output, so you may have to change the component values for  the

R/C filter and the attenuator a bit.

23 External converter

An option  "External Converter"  is available  on the  "Keying"

menu.  It can  also be controlled  by the ALT-F9/ALT-F10  keys.

If this option is enabled, a mark/space signal from an external

converter or modem chip is expected  at the CTS pin of the  COM

port.

Please note that the HamComm  interface is no longer  required,

but is highly recommended.  If  you don't use it you loose  the

SCOPE and  SPECTRUM functions  which are  also very  handy  for

precise tuning.   Connect the input  of the  converter and  the

input of  the HamComm  interface in  parallel to  the  receiver

output.

Filter  converters   for   RTTY  are   normally   designed   to

differentiate between two tones.   For CW  reception a kind  of

tone decoder is required that can tell the tone from noise.

24 External AFSK

AFSK tones  for  transmission  are normally  available  at  the

speaker output or the TxD  pin of the COM  port.  If you  don't

want to use those signals for whatever reason, you can find  an

FSK signal  at  the  DTR  pin  of  the  COM  port.    For  RTTY

transmission DTR is negative for 'mark' state and positive  for

'space' state.  In  CW mode DTR is  negative for 'no-tone'  and

positive for 'tone'.  During reception DTR is always positive.

17

HamComm 3.1                  Manual               June 10. 1996

Note that in receive  mode the RTS pin  is negative and  always

positive during transmissions.  It can thus be used to gate the

DTR signal for keying a transmitter in CW mode.

If you attach  any additional components  to these pins  please

keep in mind that the DTR/RTS signals supply the power for  the

OpAmp.

25 Signal decoding

The amplified audio signal is connected to the DSR modem status

input of the serial port where every zero crossing generates an

interrupt.   HamComm  determines the  time  between  successive

interrupts  using  the  Pcs  timer  chip  and  calculates   the

corresponding tone frequency.  Due  to the timer resolution  of

about  one  microsecond  the  result  is  quite  accurate   and

constitutes the base for all further signal processing.

For RTTY  decoding  the  tone  is  compared  to  the  currently

selected center frequency to decide wether this is a 'mark'  or

'space' signal.  The mark/space signal is sampled at the proper

time according to  the current Baud  collecting all  bits of  a

character.

For CW decoding the program has to differentiate the tone  from

noise.   To be  regarded a  valid  signal the  tone has  to  be

between the  currently selected  mark/space frequencies  for  a

certain amount  of  time.   The  program maintains  a  floating

average of  the length  of the  dots and  dashes to  adjust  to

varying speeds.

When the character is  complete it is  converted to ASCII  code

and displayed in the RX window.

26 RTTY Basics

Radio  TeleTYpe  (RTTY)  uses  two  different  tones  for  data

transmission.  They  are called  the 'mark'  and 'space'  tone,

where  mark  is  usually  the  higher  tone.    The   frequency

difference between them is called 'shift'.  Transmission  speed

is specified in 'Baud',  wich is the  number of signal  changes

per second.  Ham radio RTTY  normally uses 45 Baud and a  shift

of 170Hz on shortwave.

Characters are encoded using ASCII  or the more popular  Baudot

code.   ASCII  uses  combinations of  7  bits  to  represent  a

character while  Baudot uses  only 5  bits.   With  two  signal

states (mark and space) we can only transmit one bit at a time.

The bits of a  character are therefore  sent one after  another

starting with the least significant bit.  If the value of a bit

is 1 then it is represented by the mark tone, otherwise by  the

space tone.

The mark tone  is also used  whenever the  transmitter is  idle

e.g. there is  no character left  in the transmit  buffer.   In

front of every  character a  single space  bit is  sent.   This

'start bit'  informs the  receiver that  a   new  character  is

18

HamComm 3.1                  Manual               June 10. 1996

coming in.  Next  are the data  bits comprising the  character,

followed by a 'stop bit' of mark polarity.

State                Time -->

mark  ------   --------------------------------

|Sta|Bit|Bit|Bit|Bit|Bit|Stp|

|   | 1 | 2 | 3 | 4 | 5 |   |

space      -------------------------

The duration of a bitcell is 1/Baud.  At 50 Baud a bit is  20ms

long.

27 RTTY Decoding

To decode an RTTY signal the center frequency has to be in  the

middle between the  mark and space  tones.   Activate the  TUNE

screen from the MODE menu  or hit F9.   Tune the radio so  that

the lower tuning bar is between  the two upper tuning bars.  To

change the center frequency use the arrow keys or the mouse.

Signal Frequency: 01### Hz

0 . . . .500. . . .1000. . . .1500. . . .2000. . . .2500 Hz

Space ->   |   |  <- Mark

|

Center Frequency: 01360 Hz

On a fast PC  the SPECTRUM function (F7)  can be used for  more

precise tuning.  Adjust the radio so that the two peaks of  the

RTTY signal  match the  marker lines  for  the mark  and  space

tones.  The mark/space distance (shift) is controlled from  the

KEYING menu.

To start the decoder  select Baudot from the  MODE menu or  hit

F3.  Accurate tuning is essential  but you also have to  select

the proper speed and keying direction.  Hit Alt-S to select the

speed.  Ham  RTTY is  usually 45  baud, sometimes  50 baud  and

outside of the ham bands 50, 75 and 100 bauds are most  common.

If you  don't  know what  speed  to select  try  the  BITLENGTH

STATISTICS display (F6).

Hit the Tab key to toggle between Normal and Inverted keying.

RTTY is an asynchronous mode.   Sender and receiver don't  keep

their internal time  references in sync.   The receiver  simply

waits for  the  leading  edge of  the  startbit,  collects  the

databits according to the signal speed  and then waits for  the

next character.

28 Morse code basics

Morse code, also known as CW (continuous wave), uses  sequences

of short and long tones for  character encoding.  A short  tone

is called a 'dot', a long tone is called a 'dash' and should be

three times as long as a dot.  The gaps between the tones of  a

character are as  long as a  dot, the  gaps between  characters

19

HamComm 3.1                  Manual               June 10. 1996

should be as long as a dash.  The overall speed of a Morse code

transmission is  measured in  'words  per minute'  (WPM)  while

sending the word 'PARIS'.

Characters occuring very often in a  text are encoded as  short

sequences, seldomly  used  characters are  assigned  to  longer

sequences.  The letter 'E' for example is just one dot (.), 'T'

is one dash (-) and the letter 'Q' is dash-dash-dot-dash  (--.-

).

Morse code is  used for example  by maritime  services and  ham

radio operators.  Try the lower end of the amateur 80m, 40m and

20m bands starting at 3.5MHz, 7MHz and 14mhz.

29 Morse code decoding

To decode a CW  transmission the tone  frequency of the  signal

should closely match the  currently selected center  frequency.

Activate the TUNE screen  from the MODE menu  or hit F9.   Tune

the radio so that the upper  bar indicating the signal tone  is

lined up with the  lower bar.  To  change the center  frequency

use the arrow keys or the mouse.

Signal Frequency: 01### Hz

0 . . . .500. . . .1000. . . .1500. . . .2000. . . .2500 Hz

|

|

Center Frequency: 01360 Hz

On a fast PC  the SPECTRUM function (F7)  can be used for  more

precise tuning.   Adjust the  radio so  that the  CW signal  is

centered between the green marker lines.  To start the  decoder

select CW from the MODE menu or hit F2.

There are two problems encountered with CW decoding in HamComm.

Problem 1: Tone detection

For many  people it  is obvious  that an  RTTY signal  has  two

states, usually called mark/space or  high/low tone.  Any  data

transmission requires at least two states  for 1 bit at a  time

and CW is no  exception.  So  what are the  two states for  CW?

They are the tone and ... noise !  You may be surprised to hear

that a certain amount of noise is required by HamComm to detect

that the tone  has ended.   For  this reason  CW decoding  with

HamComm works not very well with narrow filters.

HamComm has an indicator for the tone detector output.  At  the

bottom of the RX window, just to the right of the WPM  display,

is a  small  white  dot that  jumps  up  and down:  up  =  tone

detected, down  = no  tone (noise)  detected.   Listen  to  the

signal and watch the bouncing dot.  It should follow the signal

precisely and  not  jump  around  nervously  or  stick  to  one

position.

The  input  signal  is  compared  to  the  currently   selected

mark/space tones.  To be regarded  a valid tone the signal  has

20

HamComm 3.1                  Manual               June 10. 1996

to stay in that  range for a  certain amount of  time.  So  the

behaviour of  the detector  depends on  the currently  selected

shift.

The tone detector output as displayed  by the dot is the  input

to the next stage, the character decoder.

Problem 2: Character decoding

In theory a  dash is three  times as long  as a  dot, the  gaps

within  a  character  are   dot-sized  and  the  gaps   between

characters are dash-sized.  In reality there are short and long

tones and gaps of  variable size because  CW is usually  'hand-

made.'   The  speed and  length  ratios also  change  during  a

transmission if the operator gets tired or bored.

Overall decoding quality mainly  depends on the tone  detector.

HamComm keeps a floating average of the tone duration to adjust

to speed changes.   If  the signal is  too noisy  the dots  and

dashes will get  broken into many  short ones.   The  character

decoder then gets bursts of very short tones which look like  a

high-speed CW signal and  tries to adjust.   The WPM  indicator

goes up and shows a much too high value resulting in  sequences

of 'e' and 't' characters.

Especially under noisy condition you  may get better result  if

you turn the automatic speed adjustment  off and select the  CW

speed manually.  Disable the AutoWPM  option on the SPEED  menu

and select the desired speed.

If you are not familiar with Morse code even a perfect copy may

look  like  garbage  to  you.     CW  operators  love  to   use

abbreviations for almost  everything:   'gn es  hpe cuagn'  for

example means "good night and i hope to see you again".

30 SITOR A/B

SITOR (SImplex Teleprinting Over Radio) is a telegraphy  system

used for  message exchange  between  ships and  land  stations.

There are two modes called SITOR A and SITOR B.

SITOR has been  adopted for ham  radio as  AMTOR.   Technically

AMTOR ARQ is therefore  the same as SITOR  A, AMTOR FEC is  the

same as SITOR B.

See the section on AMTOR for more information.

31 NAVTEX

The NAVTEX system on 518kHz employs SITOR B for transmission of

navigational and meteorological information.   AMTOR FEC  works

like SITOR B and can thus be used to monitor NAVTEX messages.

Please note that 518kHz is the RF frequency of the signal.   To

hear it in USB tune to 516.6kHz.  Since there are many stations

around the world all using the same frequency they have to  use

21

HamComm 3.1                  Manual               June 10. 1996

a fixed time  schedule for their  transmissions.   It may  take

quite some time until your hear a signal.

See  the  section  on  AMTOR   for  more  information  on   FEC

monitoring.

32 AMTOR

AMTOR (AMateur Teleprinting Over Radio)  is an adoption of  the

maritime SITOR  system for  Amateur  Radio by  Peter  Martinez,

G3PLX.  He  was the  first who  designed and  built a  low-cost

AMTOR code converter.  AMTOR works exactly like SITOR with  the

addition of a LISTEN mode for ARQ transmissions.

AMTOR provides two modes of communication, ARQ and FEC.  In the

ARQ (Automatic ReQuest) mode the 'information sending  station'

(ISS)  transmits   a  block   of  three   characters  and   the

'information  receiving   station'  (IRS)   replies  with   one

character for acknowledgment.   If the  block arrived intact  a

'positive acknowledgment' (ACK) is  sent and the ISS  continues

with the  next block.   Otherwise  a 'negative  acknowledgment'

(NAK) character is sent and the ISS repeats the block in error.

Because of this handshaking  only two stations can  participate

in an ARQ link.  Other stations can monitor the QSO, but  there

will be no error correction.

FEC (Forward Error  Correction) is  a broadcast  mode with  one

sender and many receivers.  The sending station transmits  each

character twice,  but not  in a  row.   Between the  first  and

second transmission  four other  characters  are sent,  so  the

second transmission is delayed by 350ms.

Speed for ARQ and FEC transmission  is always 100 Baud (=  10ms

per bit) and will be automatically selected by HamComm.

33 FEC monitoring

The procedure to monitor FEC  transmissions is very similar  to

RTTY monitoring.  Select AMTOR FEC from the MODE menu or hit F5

to activate FEC mode.  The speed is internally set to 100  Baud

so you don't have  to change the Baud  setting manually.   Make

sure that a shift of 170Hz is selected on the KEYING menu.  Set

the keying direction to 'normal' for USB, select 'reverse' when

receiving in LSB.  Tune in to the FEC signal in the same way as

described for RTTY above.

AutoUnshift should be disabled, check the  TEXT menu.  The  AFC

feature may be helpful to  compensate receiver drift for  long-

term reception.

HamComm may need  a few  seconds to get  in sync  with the  FEC

signal.   Instead  of the  Baud  indicator there  is  a  status

display in the lower left corner.  STBY (=Standby) is displayed

while HamComm  is  looking for  a  valid signal.  ERR  (=Error)

indicates  that  an  illegal   character  was  received,   TRFC

(=Traffic) indicates a valid character.

22

HamComm 3.1                  Manual               June 10. 1996

34 FEC transmission

To transmit  an FEC  signal just  activate  AMTOR FEC  mode  as

described above and hit control-T or toggle RX/TX mode from the

MODE menu.  The text to send is entered in the transmit window.

Use  the  EOT  character  (default  =  control-backspace)   for

automatic return to RX mode at the end of the text.

35 ARQ LISTEN mode

To monitor an ARQ transmission select AMTOR ARQ LISTEN from the

MODE menu or hit F4.  The  speed is internally set to 100  Baud

so you don't have  to change the Baud  setting manually.   Make

sure that a shift of 170Hz is selected on the KEYING menu.  Set

the keying direction to 'normal' for USB, select 'reverse' when

receiving in  LSB.   AFC and  AutoUnshift should  be  disabled.

Tune in to the ARQ signal in the same way as described for RTTY

above.

AMTOR ARQ sends  a block about  twice a  second (every  450ms).

The signals  are  easily  identified  by  their  characteristic

chirp-chirp sound.    It  requires some  practice  und  careful

listening to  tell wether  the station  is sending  information

blocks or acknowledgment  characters.   Information blocks  are

about 210ms long, acknowledgments only  70ms.  This results  in

slightly different sounds.   Of course only information  blocks

will be decoded.  If you are not sure try to take a look at the

signal using the SCOPE function.

In ARQ  LISTEN mode  HamComm tries  to find  valid  information

blocks to lock  on the incoming  signal.  Instead  of the  Baud

indicator there is a  status display in  the lower left  corner

indicating one of the following conditions:

STBY    (Standby)   looking for valid information blocks

LOCK    (Locked)    valid blocks found, locked on signal

TRFC    (Traffic)   valid character received

ERR     (Error)     invalid   character   received   (at   this

station)

REQU    (Request)   ISS   requests   retransmission   of   last

acknowledgment

REPT    (Repeat)    ISS  sends  same  block  again  (block  not

displayed)

IDLE    (Idle)      ISS has no text to send

36 ARQ transmission

Calling cq:

FEC is  used for  calling cq  in AMTOR.   Keying  direction  is

'normal' in USB and 'reverse' in LSB.  Switch to FEC mode  (F5)

and hit control-T to  start the transmitter.   Let it idle  for

half a minute before  entering text.  The  signal of an  idling

FEC station has a special sound that is immediately  recognized

by experienced AMTOR operators.  It also allows other  stations

to tune and synchronize to your signal.

23

HamComm 3.1                  Manual               June 10. 1996

A typical cq call looks like this:

CQ CQ CQ DE DL5YEC DL5YEC DL5YEC SELCAL DYEC DYEC DYEC

Repeat the text four or five times, then terminate with 'PSE K'

and go back to RX mode.  The EOT character  (control-backspace)

can be used to automatically switch  TX mode off.  If there  is

no reply, repeat the procedure.

The other station can  call you back using  either FEC or  ARQ.

If it  comes  back in  ARQ  you have  to  switch to  AMTOR  ARQ

manually using the MODE menu or ALT-F4.  When your station  has

received it's own SELCAL as defined  in HC31.CFG it will  start

to reply with  acknowledgments and the  other station 'has  the

keys'.

In AMTOR ARQ  the character sequence  '+?' is used  by the  ISS

(information sending station)  to turn  the link  around.   The

change-over is done automatically, so whenever you receive  the

magic '+?' sequence your station becomes the ISS.  Don't forget

to terminate your  text with  the same  sequence.   Transmitted

text will appear in the RX window in a different color.

In ARQ there  is also  a way  to 'seize'  the link.   Just  hit

control-T while you are the IRS (information receiving station)

and a change-over procedure is initiated.  This feature  should

only be used for urgent messages like 'TX IS BURNING'.

To shut down the link use  the EOT character while you are  the

ISS  Transmission  will also  be disrupted  immediately if  any

other operating mode like CW, FEC or BAUDOT is selected.

Calling an ARQ station:

If you want  to answer a  cq call or  contact one  of the  many

AMTOR mailboxes,  you  have  to  specify  the  SELCAL  of  that

station.  Select item 'Define SELCALL'  from the TEXT menu  and

enter the four-letter selective call.  Switch to AMTOR ARQ mode

(ALT-F4) and  hit  control-T.    Your  station  will  start  to

transmit special information  blocks containing  the SELCAL  of

the other station.   Transmission will  stop if  there isn't  a

reply within a minute.  If  the link is made, operation is  the

same as described above.

37 Error correction

There is  no  communications  protocol that  can  guarantee  an

error-free data transmission.   The differences  are mainly  in

the reliability of the  error detection scheme  in use.   Newer

modes like packet radio  and PACTOR add a  CRC checksum at  the

end of a data block.  The probability that a transmission error

will go by unnoticed is very low.

With the AMTOR FEC and ARQ modes there is no checksum.  The  7-

bit AMTOR code is basically the 5-bit Baudot code with two bits

added to  each  character.   With  seven  bits  there  are  128

possible combinations, but  only the  35 codes  are used  which

have four bits high and three  bits low.  An error is  detected

if any of the other codes is received.

24

HamComm 3.1                  Manual               June 10. 1996

If two or  more bits get  mutilated the result  may be  another

valid character.  AMTOR links are  not error-free,  there  will

be an undetected transmission error every now and then on a bad

link. This isn't really  a problem as  normally there are  more

typos than transmission errors.

38 Clock correction

One reason why AMTOR ARQ is  much more robust than RTTY is  the

synchronisation.   Once the  link is  established the  internal

clocks are kept in  sync so both  stations know very  precisely

when the next bit will arrive.

The ARQ station initiating a link  is called MASTER, the  other

station is SLAVE.  This is not to be confused with ISS and IRS.

The SLAVE will constantly adjusts its clock to the MASTER clock

until the link is terminated.

The time  references  of  the  commercial  SITOR  stations  are

accurate to about +/-30ppm (parts  per million).  When  HamComm

was tested on many different PCs  it was found that their  xtal

oscillators sometimes deviate up to 400ppm.

One may ask  why the  clocks have to  be so  accurate when  the

SLAVE has to adjust  to the MASTER  anyway.  Reception  suffers

from noise and fading.  If the signal is bad there is not  much

phasing information  available to  adjust  the clock,  so  both

stations would  quickly run  out of  sync. Accurate  timing  is

therefore essential to keep the link going, especially when the

conditions are poor.

So the clock  has to  be corrected.   To  find the  'clockcorr'

value for a  specific PC, an  ARQ reference  signal is  needed.

This could be an AMTOR station but it is usually easier with  a

SITOR signal. If you have a receiver that can tune outside  the

amateur bands,  try to  find a  SITOR station  around  2020kHz,

4200kHz, 6320kHz  or 8400kHz.   We  need a  strong signal  with

information blocks.  Select AMTOR ARQ LISTEN mode (F4) and tune

to the signal.

A correction counter is shown in  the lower left corner of  the

RX window, to the right of link status display.  The correction

counter will change as soon as HamComm is locked on the  signal

and starts to decode.  The link status now displays TRFC, REPT,

REQU or IDLE.  Hit the ESC  key to reset the counter and  watch

the it.  We need  to know how many  seconds it takes until  the

counter has  reached  the  value  10 or  -10.    If  there  are

interruptions in the transmission you  will have to start  over

again.  Repeat the procedure a  few times to make sure you  got

the right value.   If it takes more  than two minutes to  reach

the final count your PC happens to be accurate enough.

The clockcorr value is the number of seconds times 100.  If the

count is negative  then the  clockcorr value  is also  negative

Example: The counter reached -10 in 43 seconds.  The  clockcorr

value is therefore -4300

25

HamComm 3.1                  Manual               June 10. 1996

The clockcorr setting  has to be  changed in the  configuration

file HC31.CFG.  Using your favourite editor, find the line "set

clockcorr 0"  and change  it to  the value  calculated  before.

With the right value the correction count will be stable during

ARQ reception.

39 Macro keys

A macro  key  can  be used  to  replay  a  previously  recorded

sequence of  keystrokes.   The digits  on the  top row  of  the

keyboard are used as macro keys while the ALT key is depressed.

They are normally located  right above the  QWERTY... row.   Do

not confuse these keys with the numeric keypad!

In the beginning there are no keystrokes assigned to the  macro

keys, so pressing ALT-1, ALT-2...ALT-9 or ALT-0 has no  effect.

To start recording, select item 'Macro recorder' from the  FILE

menu.  A message box will appear confirming that recording  has

been started.  When the ENTER key is hit, the message box  will

go away and from now on  up to 250 keystrokes will be  recorded

along with  normal  processing.    A  blinking  "! RECORDING !"

message is shown  at the lower  right corner of  the screen  to

remind you that the recorder is still active.

Now hit one of the macro  keys to stop recording and to  assign

the keystroke sequence  to that key.   This  also replaces  any

previous assignment.  A message box will appear to confirm that

macro recording has terminated.   Every time  the macro key  is

hit the recorded keystroke sequence will be replayed just  like

it had been typed again.

To terminate  recording without  changing  any macro  key  just

select 'Macro recorder' again  from the FILE  menu.  A  message

box will  appear  to  confirm that  macro  recording  has  been

canceled.

To clear  a macro  key start  recording in  the usual  way  and

directly after that hit the key to be cleared.  This will erase

any previous recording  assigned to that  macro key.   Again  a

message box will confirm the result.

40 SHIP/SYNOP decoder

Weather reports are transmitted by many stations troughout  the

world, 24 hours  a day.   If  you live  in europe  try to  copy

Quickborn Meteo  (near  Hamburg,  Germany) on  4583,  7646  and

11638kHz, or Bracknell (UK) around  4488kHz.  In north  america

try CFH (Halifax,  Nova Scotia) on  4271, 6496.5 and  10536kHz.

There are many others between 4000 and 5000kHz.   Transmissions

are normally in Baudot, 425Hz shift, 50 or 75 Baud.

The SYNOP format is used for  reports from land stations,  SHIP

format is  used  for  reports from  ships  and  other  maritime

vessels.   The messages  include data  about temperature,  wind

speed/direction,   cloud   cover,   precipitation,   dew-point,

pressure and other  meteorological information.   SHIP  reports

also include the current position of the vessel.

26

HamComm 3.1                  Manual               June 10. 1996

Land stations are  identified by a  five-digit station  number.

These are fixed stations, so their geographical position is not

transmitted with a  SYNOP report.   To give you  a better  idea

where the reporting  station is located,  HamComm comes with  a

list  of  about   10000  station  numbers,   their  names   and

geographical positions.   The program  will automatically  pick

the correct entry from this list while decoding a SYNOP report.

A typical undecoded message looks like this:

zczc 548

sien42 edzw 141500

aaxx 14154

01465 42889 42715 10084 20022 40159 52033 81048=

02060 41480 40000 11088 21113 40060 52035 72272

83530 333 83694=

nnnn

This is what you get if the SHIP/SYNOP decoder is switched off.

The first line of a message starts with zczc and a  three-digit

transmission sequence  number.   The second  line is  a  header

describing the following information.  Here 'sien42'  indicates

a "synoptic report at intermediate hours" for northern  europe,

'edzw' is the international  four-letter location indicator  of

the station originating or compiling the bulletin and  '141500'

means 3pm UTC on day  14 of the current  month.  'aaxx' is  the

indicator for SYNOP reports, 'nnnn' marks the end of a message.

There  are  other  formats  in  use  for  different  kinds   of

information.  Currently HamComm only supports decoding of  SHIP

and SYNOP reports.  A message  may contain one or more  reports

separated by = characters.  Explaining the reports in detail is

beyond the scope of this manual.

With the WX decoder switched on  (from the TEXT menu) the  same

message looks like this:

zczc [start] 548 [message 548]

sien42 [Synoptic reports at intermediate hours (SYNOP, SHIP)]

[Northern Europe]

edzw [Offenbach (MET/COM Centre)]

141500 [day:14 UTC:1500]

aaxx [SYNOP]

14154 [day:14 UTC:1500]

[Wind speed obtained from anemometer (knots)]

01465 [Norway, 58024'N 008048'E TORUNGEN (LGT-H)]

42889 [manned] [cloud height:2000-2500m] [visibility:75km]

42715 [cloud cover:4/8] [wind dir:270 deg, speed:15]

10084 [air temp:+8.4]

20022 [dew-point temp:+2.2]

40159 [pressure at sea level:1015.9hPa]

52033 [pressure:increasing] [change in 3h:3.3hPa]

81048 [cloud info]

=

02060 [Sweden, 68041'N 021032'E NAIMAKKA]

41480 [manned] [cloud height:300-600m] [visibility:30km]

40000 [cloud cover:4/8] [wind dir:calm, speed:0]

27

HamComm 3.1                  Manual               June 10. 1996

11088 [air temp:-8.8]

21113 [dew-point temp:-11.3]

40060 [pressure at sea level:1006.0hPa]

52035 [pressure:increasing] [change in 3h:3.5hPa]

72272 [past wx: snow, or rain & snow mixed,

cloud cover > 1/2 of sky]

[wx now: Snow]

83530 [cloud info]

333 [section 3]

83694 [clouds:3/8, stratocumulus, 1000-1500m]

nnnn [EOM]

HamComm displays  the  incoming text  just  as before.  The  WX

decoder watches  the  characters  go by,  waiting  for  certain

keywords like zczc  (start of msg),  aaxx (SYNOP report),  bbxx

(SHIP report)  and nnnn  (end of  msg) to  synchronize with  an

incoming message.   If  it thinks  it knows  what the  text  is

about,  it  starts  to  insert  comments.    The  comments  are

surrounded   by   square   brackets   and   therefore    easily

distinguished  from  the  normal  text  (there  are  no  square

brackets in the Baudot character set).

There are other ways to build  a SHIP/SYNOP decoder.  It  could

collect all data for a report and then display the result in  a

nicely formatted way  However, there is a problem.  The kind of

RTTY  transmission  used  for   these  reports  has  no   error

correction,  not   even  error   detection.     It's  hard   to

automatically classify  a  message  as  ok  or  garbled.    For

example: if the program says there  are +30 degrees celsius  in

Greenland and it's december, you  will hopefully not buy  that.

HamComm has no  idea where Greenland  is and  not the  foggiest

about the climate there, so it can't catch that type of error.

The program displays the incoming text as received, so you  can

see if there is garbage coming in.  Usually it is quite obvious

if the input is bad.  With  a WX decoder working like a  'black

box' it  is sometimes  very hard  to  see how  it came  to  its

conclusions and what to believe or not.

41 Scope function

In RX  mode the  tone decoder  routine calculates  the  current

signal frequency for use  by other parts of  the program.   The

SCOPE function uses this value to display a graph of the  input

frequency vs. time.  This is the blue line on the SCOPE screen.

The tone decoder also  maintains a floating average  frequency.

The effect is very similar to a low-pass filter and is used for

noise reduction.   The result is  displayed at the  top of  the

screen as a red line.

The solid  green  line  marks  the  currently  selected  center

frequency and the dotted  lines above and  below show the  mark

and space tones.  For RTTY reception the center line should  be

in the  middle  between the  mark  and  space tones.    For  CW

reception the signal  should match the  center frequency.   All

signals outside  the  area  marked  by  the  dotted  lines  are

ignored.

28

HamComm 3.1                  Manual               June 10. 1996

The display can be controlled with the following keys:

F1  display help screen

F10  activate menu bar

'+'  toggle display of decoded signal

'*'  toggle grid display

'B'  toggle position of red line

HOME  reset sample rate to 1ms

PG-UP  sample rate faster

PG-DOWN  sample rate slower

ENTER  hold display immediately

SPACE  return to RX/TX screen

The right mouse  button holds  the display  at the  end of  the

current sweep.  The  left mouse button  activates the menu  bar

just like F10.

The  red  line  (input  signal  after  low-pass)  is   normally

displayed at the top  of the screen to  not interfere with  the

blue line.   In its second  position it will  overlay the  blue

line to compare the two signals.

Display of the decoded signal is normally disabled.  It can  be

found at  the bottom  of the  screen and  shows the  mark/space

states for RTTY and tone/no-tone states for CW reception.

With some practice it is quite easy to determine the signal you

are listening to by watching the SCOPE display.  CW, AMTOR  ARQ

and FEC,  Packet  Radio  and  RTTY  show  their  characteristic

patterns and you can also  watch unstable VFOs sweeping  slowly

across the band.

42 Spectrum function

On entry to the Spectrum function a graphics mode according  to

the video card and monitor in use is activated.

Every millisecond the current input frequency is checked.   For

every frequency there is a counter and the one that corresponds

to the current  frequency is incremented.   All counter  values

are displayed as vertical  lines with each  sweep from left  to

right  The higher the count, the longer the line.  What you see

on the screen is therefore not the true audio spectrum but  the

result at the output of the tone decoder routine.  An opamp and

a few lines of code can't substitute a real spectrum analyzer.

When set to 1Hz per pixel  SPECTRUM has the best resolution  of

all display functions.  For example,  the actual shift used  by

an RTTY station can be determined quite accurately.

29

HamComm 3.1                  Manual               June 10. 1996

The display can be controlled with the following keys:

F1  display help screen

F10  activate menu bar

'-'  toggle raw/filtered data

'+'  toggle 'shaddow' (color displays only)

'*'  toggle dot/line mode

HOME  reset, start at 200Hz, 3Hz per pixel

PG-UP  zoom in

PG-DOWN  zoom out

LEFT  lower frequencies

RIGHT  higher frequencies

ENTER  hold display

SPACE  return to RX/TX screen

The right mouse  button holds  the display  at the  end of  the

current sweep.  The  left mouse button  activates the menu  bar

just like  F10.   If you  have a  slow PC  then don't  use  the

SPECTRUM function,  you might  get  frustrated.   There  simply

isn't enough CPU power to keep the display in motion.  AT class

maschines and real fast XTs should be ok.

The SPECTRUM function has been found to be quite  entertaining,

especially with a color display.  Next time you got visitors in

your shack  who don't  know what's  going  on just  start  this

function, turn the lights low and tune across a crowded band.

43 Bitlength statistics

The BITLENGTH function is  a graphics display  of the MARK  and

SPACE pulse length as detected by  HamComm.  It's main  purpose

is to quickly get an idea about the speed of the signal.

With every transition of the  decoded MARK/SPACE signal a  high

or low  pulse  ends.   The  upper  histogram  shows  the  pulse

duration statistics  of the  high pulses,  the lower  histogram

shows the statistics of the low pulses.

Before using this function the center frequency must be set  to

the center  between the  MARK and  SPACE tones,  otherwise  the

pulse width display will suffer from distortion.

The shortest pulses of an RTTY signal are normally equal to the

length of one bit.  Therefore  the leftmost peak indicates  the

Baud of the incoming signal.   A noisy signal produces lots  of

short spikes.  They tend to  accumulate at the left end of  the

histogram and should be ignored for Baud analysis.

The BITLENGTH  function tries  to identify  the leftmost  valid

peek automatically,  but  this  may fail  with  noisy  signals.

Below each histogram the Baud corresponding to the highest peak

is displayed.  There is also a MARKER that can be moved  freely

using the LEFT and RIGHT keys.

30

HamComm 3.1                  Manual               June 10. 1996

The display can be controlled with the following keys:

F1  display help screen

F10  activate menu bar

PG-UP  zoom in

PG-DOWN  zoom out

LEFT,RIGHT  move marker

CONTROL-LEFT  move marker, fast mode

CONTROL-RIGHT   "     "      "    "

ENTER  hold display

SPACE  return to RX/TX screen

The marker can  also be controlled  by moving  the mouse  while

holding  down  the  right  mouse  button.    The  Baud   number

corresponding to the marker position is displayed at the  upper

left corner of the screen.

The left mouse button activates the menu bar just like F10.

44 Interface checkout

So you  have  build  the  interface  and  made  the  neccessary

connections to the radio.  You fire up the program and you  see

- nothing.

To find out what is going wrong let's start at the serial  port

HamComm needs to know two parameters about a COM port:

1. the port address

2. the IRQ number

The  port  address  and  IRQ  number  for  COM1  and  COM2  are

standardized  and  therefore  predefined  in  HC31.CFG  to  the

following values:

addr   IRQ

COM1   3F8h    4

COM2   2F8h    3

For other serial ports there is  no real standard, so COM3  and

COM4 are not defined in the supplied HC31.CFG.  Undefined ports

can not be selected from the PORT menu.

45 Port address

Start the  program and  select entry  'HamComm' from  the  INFO

menu.  A small window  appears displaying, among other  things,

the port address  and IRQ  of the  serial ports  as defined  in

HC31.CFG.  If  the values for  the port you  want to use  don't

match the hardware, correct HC31.CFG with a text editor.

Remove the interface for now.  Select the correct port from the

port menu, then hit F3 to  activate the RX/TX screen.  Using  a

voltmeter check the RTS and DTR outputs of the serial port with

respect to the GND pin.  RTS should have a negative voltage and

DTR should be positive.

31

HamComm 3.1                  Manual               June 10. 1996

Now switch to transmit  (TX) mode.  This  can be done from  the

MODE menu but it's easier to hit control-T.  When switching  to

TX mode both RTS and DTR reverse their polarity, so RTS  should

now be positive and DTR should be negative.

If they don't, here is a list of what might be wrong:

- the voltmeter is broken.

- you are measuring at the wrong socket.

The serial ports of a PC are normally

25-pin or 9-pin male SUB-D sockets.

- you are measuring at the wrong pins.

4     7                              4   5

RTS   GND                            DTR GND

----1-----|-----|-----------13---     ----1-----|-|----

\         |     |               /     \         | |   /

\  o o o o o o o o o o o o o  /       \  o o o o o  /

\  o o o o o o o o o o o o  /         \  o o o o  /

\             |           /           \   |     /

14-----------|---------25             \6-|---9/

DTR                         RTS

20                          7

- you have selected the wrong com port. Check the PORT menu.

- the port address as defined in HC31.CFG for this port is not

correct.  Check the INFO menu and HC31.CFG.

- the com port has a different address from what you think.

The address can often be selected with jumpers on the

serial card.  Check the documentation for your computer.

On some newer PCs and notebooks the com port address can

be changed or the port can be disabled with a setup

program.

- if only one of the signals (RTS or DTR) changes polarity,

the line driver for the other signal may be broken.

46 Op-Amp power supply

Now let's take a look at the interface.  By far the most common

problems are:

- wrong or missing connections

- solder bridges and cold solder joints

If you think the  problem is with the  interface take the  time

and carefully inspect it under a magnifying glass.

If that doesn't work, proceed as follows.

Switch off the computer and connect the interface to the serial

port. Don't  connect it  to the  radio yet,  connect the  audio

input to  GND. If  possible the  interface should  be  directly

plugged to  the com  port. Any  cable between  the PC  and  the

interface should be avoided, it may degrade the signal quality.

32

HamComm 3.1                  Manual               June 10. 1996

Switch the PC on, start HamComm, select the right port from the

PORT menu and hit F3 for the RX/TX screen.  Check that RTS  and

DTR still change  polarity when you  toggle between  RX and  TX

mode.

Check the supply voltages  at the op-amp  with respect to  GND.

The positive voltage  at pin  7 should  be +5V  or higher,  the

voltage at pin 4 should be -5V or lower (more negative).

+V out

8  7  6  5

|  |  |  |    Operational Amplifier

|------------|

|            |  LM741 or TL071

|            |

|-|          |  DIL-8 package, top view

|          |

|-|          |   +V = positive supply

|            |   -V = negative supply

|            |   -I = inverting input

|------------|   +I = non-inverting input

|  |  |  |    out = output

Pin 1  2  3  4

-I +I -V

Check the voltages both in RX and TX mode at the op-amp.   Here

they should  NOT  change  polarity, otherwise  the  IC  may  be

permanently damaged.   If  they do  or if  the readings  change

significantly in RX and TX mode, check the wiring and  polarity

of the four diodes between the  PC and the op-amp.  Also  check

the two capacitors used for buffering the supply.

Although only a current of a few milliamperes is needed for the

op-amp, some  com ports  can not  deliver the  required  supply

voltages.   You may  try  to find  an  op-amp that  draws  less

current  but  the  easiest  solution  is  to  use  two   9-Volt

batteries:

bat 1      bat 2

||         ||

op-amp pin 7 <---||----O----||---> op-amp pin 4

+||-   |   +||-

GND

If the power supply is ok  we will now try  to find out if  the

op-amp works  correctly.    First  check  the  voltage  at  the

inverting and non-inverting inputs.  Both should read 0 or VERY

close to 0 volts with respect to GND.  If they don't, there  is

probably a wiring error or the chip is broken.

Temporarily connect a  resistor of about  1M (megaohm)  between

the non-inverting input  (pin 3) and  the positive supply  (pin

7).   Be very  careful not  to short  any pins.   The  op-amp's

output should go as high as it can go, which is normally  about

1 volt below the positive supply.

Remove the resistor and  connect it between  the input (pin  3)

and the negative supply (pin 4).   The output should now be  as

33

HamComm 3.1                  Manual               June 10. 1996

low as it can go, about  1 volt above (more positive) than  the

negative supply.

Check the output signal directly at the DSR pin of the com port

to make sure that the wiring is ok.  This is only a very simple

test wich the op-amp will only fail if it is badly damaged.

The output  of  the op-amp  will  also be  positve  or  negativ

without the resistor. This is normal and doesn't mean that  the

chip is broken.

47 IRQ number

Now connect the input of the  interface to the audio output  of

the receiver.  The  op-amp is quite  sensitive and should  work

even with very low  signals.  Some radios  have a socket for  a

tape recorder where the amplitude is independent of the  volume

setting.  Try that first.

Tune in to some strong signal,  preferably to a constant  tone.

At the DSR pin  of the com  port there should  now be a  square

wave signal.  The amplitude at DSR should have at least the +/-

5V swing required to reliably drive an RS-232C input.

Start  HamComm  to  check  if  the  signal  at  DSR   generates

interrupts.  Select the right port  from the PORT menu and  try

the TUNE, SCOPE or SPECTRUM function from the MODE menu.

If all of these functions seem to be dead, HamComm doesn't  get

any hardware interrupts  from the  serial port.   Select  entry

'HamComm' from the INFO menu and check the displayed IRQ number

for the port you want to use.  If it's wrong, specify the right

one in HC31.CFG.

Make sure that the port really uses the IRQ you think it  does.

Many serial cards have jumpers or  small switches for IRQ  line

selection.  Some  newer PCs and  notebooks can do  this from  a

setup program.

Please note, that an IRQ line can only be used by one device at

a time.  For example, if  you have a mouse  on IRQ 4 you  can't

use a COM port on the same IRQ at the same time.

34

HamComm 3.1                  Manual               June 10. 1996

48 Appendix A - Key assignment summary

--- All Windows ---

F1                  display help text

F2                  RX/TX window, CW mode

ALT-F2              RX/TX window, ASCII mode, 7 bit

F3                  RX/TX window, Baudot mode

ALT-F3              RX/TX window, ASCII mode, 7 bit

F4                  RX/TX window, AMTOR ARQ Listen

ALT-F4              RX/TX window, AMTOR ARQ mode

F5                  RX/TX window, AMTOR FEC mode

F6                  Bitlength display

F7                  Spectrum display

F8                  Scope display

F9                  Tune window

F10                 Menu bar

ALT-1               execute macro 1

ALT-2               execute macro 2

ALT-3               execute macro 3

ALT-4               execute macro 4

ALT-5               execute macro 5

ALT-6               execute macro 6

ALT-7               execute macro 7

ALT-8               execute macro 8

ALT-9               execute macro 9

ALT-0               execute macro 0

ALT-F5              WX decoder off

ALT-F6              WX decoder on

ALT-F7              AFC off

ALT-F8              AFC on

ALT-F9              external converter off

ALT-F10             external converter on

ALT-C               Callsign decoder

ALT-X               Terminate programm

--- TUNE Window ---

HOME                reset center frequency

LEFT                decrease center frequency

RIGHT               increase center frequency

SPACE               return to RX/TX screen

--- RX/TX Window ---

SHIFT-F1            Standardtext 1

SHIFT-F2            Standardtext 2

SHIFT-F3            Standardtext 3

SHIFT-F4            Standardtext 4

SHIFT-F5            Standardtext 5

SHIFT-F6            Standardtext 6

SHIFT-F7            Standardtext 7

SHIFT-F8            Standardtext 8

SHIFT-F9            Standardtext 9

35

HamComm 3.1                  Manual               June 10. 1996

SHIFT-F10           Standardtext 10

LEFT                move TX cursor left, 1 character

RIGHT               move TX cursor right, 1 character

UP                  move TX cursor up, 1 line

PAGE-UP             move TX cursor up, 1 page

DOWN                move TX cursor down, 1 line

PAGE-DOWN           move TX cursor down, 1 page

HOME                move TX cursor to first line

END                 move TX cursor to last line

INSERT              toggle insert mode on/off

DELETE              delete character under cursor

BACKSPACE           delete character left to cursor

TAB                 toggle NORMAL/REVERSE keying direction

ENTER               transfer text line to transmit buffer

CONTROL-A           move TX cursor to beginning of line

CONTROL-B           AMTOR details on/off

CONTROL-D           insert time and date

CONTROL-E           move TX cursor to end of line

CONTROL-F           open/close logfile

CONTROL-H           same as BACKSPACE

CONTROL-I           same as TAB

CONTROL-L           toggle AutoUnshift on/off

CONTROL-M           same as ENTER

CONTROL-P           toggle transmit monitoring on/off

CONTROL-S           send textfile

CONTROL-T           toggle transmit/receive mode

CONTROL-W           toggle Line/Word mode

CONTROL-X           clear TX window line

CONTROL-Z           insert time

CONTROL-LEFT        roll RX window backwards, 1 line

CONTROL-HOME        roll RX window backwards, 1 page

CONTROL-RIGHT       roll RX window forewards, 1 line

CONTROL-END         roll RX window forewards, 1 page

CONTROL-PAGE-UP     resize RX/TX window

CONTROL-PAGE-DOWN   "      "     "

--- SCOPE Display ---

'+'                 toggle display of decoded signal

'*'                 toggle grid display

'B'                 toggle position of red line

HOME                reset sample rate

PAGE-UP             sample rate faster

PAGE-DOWN           sample rate slower

ENTER               hold display immediately

SPACE               return to RX/TX screen

36

HamComm 3.1                  Manual               June 10. 1996

--- SPECTRUM Display ---

'-'                 toggle raw/filtered data

'+'                 toggle 'shaddow' (color displays only)

'*'                 toggle dot/line mode

HOME                reset, start at 200Hz, 4Hz per pixel

PAGE-UP             zoom in

PAGE-DOWN           zoom out

LEFT                lower frequencies

RIGHT               higher frequencies

ESC                 clear display

ENTER               hold display

SPACE               return to RX/TX screen

--- BITLENGTH Display ---

PAGE-UP             zoom in

PAGE-DOWN           zoom out

LEFT                move marker left

RIGHT               move marker right

CONTROL-LEFT        move marker left, fast mode

CONTROL-RIGHT       move marker right, fast mode

ENTER               hold display

SPACE               return to RX/TX screen

37

HamComm 3.1                  Manual               June 10. 1996

49 Appendix B - Baudot code table

-- Standard --      ---- Extended ----

Code    Upper   Figures     Lower   Punctuation

03      A       -           a       _

25      B       ?           b       n.a.

14      C       :           c       ;

09      D       WRU         d       n.a.

01      E       3           e       n.a.

13      F       %           f       `

26      G       @           g       }

20      H       #           h       {

06      I       8           i       n.a.

11      J       *           j       n.a.

15      K       (           k       [

18      L       )           l       ]

28      M       .           m       >

12      N       ,           n       <

24      O       9           o       ~

22      P       0           p       n.a.

23      Q       1           q       !

10      R       4           r       $

05      S       '           s       "

16      T       5           t       n.a.

07      U       7           u       &

30      V       =           v       |

19      W       2           w       n.a.

29      X       /           x       \

21      Y       6           y       ^

17      Z       +           z       n.a.

04      SPC     SPC         SPC     SPC

08      CR      CR          CR      CR

02      LF      LF          LF      LF

31      LET     LET         LET     LET

27      FIG     FIG         FIG     FIG

00      NIL     NIL         NIL     NIL

CR    Carriage Return

LF    Line Feed

LET    Select letters

FIG    Select figures

SPC    Space

NIL    Select upper/lower, fig./punct.

38

HamComm 3.1                  Manual               June 10. 1996

50 Appendix C - Interface schematics

RS-232C

Receive Circuit                                         25  9

pin pin

IC1 TL071                   D1

or LM741 -------*--------*-I<--*--< DTR 20  4

|      |        | D2  |

3 |\ |      |      ---->I---

>------||---*-----|+\ 7     |      | |

C1   |     |  \______|______|_|________> DSR  6  6

0.1uF  |   2 |  /6     |      | | D3

|  ---|-/       |      | --I<--

Signal        |  |  |/ |4     |      |   D4  |

from          /  /     ---*---|------*--->I--*--< RTS  4  7

Receiver    R1\  \R2      |   |

100K/  /100K    |   | +

\  \       === === C2,C3

|  |       === === 10uF

|  |      + |   |  16V

>-----------*--*--------*---*-----------------> GND  7  5

Transmit Audio Circuit

(omit for receive-only)  R4       R5         R6

15K      15K        10K

--------*--/\/\--*--/\/\--*-*--/\/\--< TxD  2  3

C4      |       |        |        | |

0.1 uF   / R3    |        |      D5| |D6

<----| |---->\ 10K  === C5   === C6    - v

MIC         / var. ===.022  ===.022   ^ -

Socket      |       |  uf    |  uF    | |

<------------*-------*---*----*--------*-*--------> GND  7  5

PTT Circuit

C      (omit for receive-only)

PTT <-------------------\

Q1     \I B       R7     D7

2N2222  I--------/\/\----I<-----< RTS  4  7

/I         1K

v

| E

<-------------------*-------------------------> GND  7  5

39

HamComm 3.1                  Manual               June 10. 1996

FSK Circuit

(omit for receive-only)

C      (optional for transmit)

FSK <-------------------\

Q2     \I B       R8     D8

2N2222  I--------/\/\----I<-----< DTR 20  4

/I         1K

v

| E

<-------------------*-------------------------> GND  7  5

D1-D8 = 1N914 or 1N4148

40

HamComm 3.1                  Manual               June 10. 1996

51 Appendix D - Interface supplier

Ready-to-run HamComm-Interfaces  can  be  ordered  from  Dieter

Dippel, DF4RD.    They  are build  with  SMD  (Surface-Mounted-

Device) components and  housed in a  case similar  to a  mouse-

adapter.  There are two versions available for 9-pin or  25-pin

COM ports.

__                   __                 __

_|  |_________________|  |_

| |                       | |

COM-Port   | |      SMD-HamComm      | |   RX/TX      app.

9-pin female | |       Interface       | | 9-pin male   35mm

|_|   _________________   |_|

|__|                 |__|                __

|       app. 60mm       |

__                                      __

_|  |_____

| |         \

| |           \          __

| |             \_______|  |_

| |                        | |

COM-Port   | |      SMD-HamComm       | |   RX/TX     app.

25-pin female | |       Interface        | | 9-pin male  55mm

| |               ______   |_|

| |             /       |__|

| |           /

|_|   _____ /              |

|__|                     |                __

|

|       app. 60mm        |

The interface also works with SSTVFAX, PKTMON, EASYFAX and  the

well-known  JVFAX  fax/sstv  program  by  Eberhard  Backeshoff,

DK8JV.

For more information please request  an updated price list  and

terms of delivery from

Dieter Dippel, DF4RD

Fenitzerstr. 33

D-90489 Nuernberg

Phone/Fax: (0) 911 / 55 92 96  between

Mo. - Fr. 18:00 and 20:00 german local time

or 17:00 and 19:00 UTC

41

HamComm 3.1                  Manual               June 10. 1996

52 Index

A                                 F

AAB .......................13     F1 .........................7

AFC .......................12     FEC .......................10

AFSK ......................17     FEC monitoring ............22

AFSK output ...............17     FEC transmission ..........23

AMTOR .....................22     File menu ..................7

AMTOR ARQ .................10     Floppy .....................5

AMTOR FEC .................10     Function keys ..............7

AMTOR Listen mode .........10

ARQ .......................10     H

ARQ LISTEN mode ...........23

ARQ transmission ..........23     Harddisk ...................5

ASCII mode ................10     HC31.CFG ...............6; 15

AutoUnshift ...............13     Help system ................7

AutoWPM ...............11; 21

I

B

Info menu .................15

Baud selection ............11     Installation ...............5

Baudot .....................9     Interface .................31

Bit Length display ........11     Interface schematics ......16

Bitlength statistics ......30     Introduction ...............4

Blank lines ...............13     IRQ .......................34

C                                 K

Callsign decoder ..........15     Keying menu ...............12

CGA ........................5

Character mode ............14     L

Clear TX buffer ...........12

Clock correction ..........25     Laptop .....................6

ClockCorr .................15     LCD ........................6

COM port selection ........12     License ....................4

Config files ...............9     Line mode .................14

Converter, external .......17     Locator ...................15

CW .................9; 18; 19     Logfile .................7; 8

logrxlabel .................8

D                                 logtxlabel .................8

Define text ...............13     M

Desqview ...................5

Diddle ....................12     Macro files ................8

Macro keys ................26

E                                 Macro recorder .............8

Menu bar ...................7

EGA ........................6     Mode menu ..................9

Error correction ..........24     Morse code .............9; 19

ESC ........................7     Morse decoding ............20

Exclusion of warranty ......5     Mouse ......................6

Extended Baudot ...........14     Multitasking ...............5

External AFSK .............17

External converter ....12; 17

42

HamComm 3.1                  Manual               June 10. 1996

N                                 Shift setting .............12

SHIP/SYNOP ............15; 26

NAVTEX ....................21     Signal decoding ...........18

Normal ....................12     SITOR A/B .................21

Speaker ...............12; 17

O                                 Spectrum display ..........11

Spectrum function .........29

Operational Amplifier .....16     Speed menu ................11

Options menu ..............15     Startup ....................6

OS/2 .......................5     SuperVGA ...................6

System requirements ........5

P

T

Pactor Listen mode ........10

Port address ..............31     Text menu .................13

Port menu .................12     Transmit circuit ..........17

Printer ....................8     Tune display ..............11

PRN device .................8     TX buffer .................14

PTT circuit ...............16

U

Q

Update .....................4

QSO count .................13

V

R

VGA ........................6

Receive circuit ...........16     Video adapter ..............5

Reverse ...................12

RTTY ......................18     W

RTTY Basics ...............18

RTTY decoding .............19     Windows ....................5

Word mode .................14

S                                 WPM selection .............11

WX decoder ................14

Scope display .............11

Scope function ............28     X

Screen layout ..............7

Selection ..................7     XT .........................5

Send text .................13

Send text file .............9

43