OpenPF Plot-File Standard

Version 1.0  June 1, 1995

OpenPF is an open, nonproprietary standard for computer

files that contain electromagnetic-field data.  OpenPF is

intended for files created by antenna-analysis programs and for

files containing measurements.  OpenPF files can be used to

archive or transport data.  Programs can use the files to plot

fields and radiation patterns on a screen, printer, or plotter.

The OpenPF file format is extensible.  Data is organized

into blocks that begin with a block-type identifier followed by

block length.  This structure allows a file parser to skip and

ignore blocks it's not designed to handle, including block types

defined in future revisions of OpenPF.

The blocks defined below provide for 2-D relative far-

field data (azimuth and elevation radiation patterns) and 3-D

absolute near- and far-field data.  3-D radiation patterns can

be represented by a set of 2-D data blocks.

The file format enables a single file to contain near-

and far-field data, absolute and relative data, azimuth and

elevation data, low- and high-resolution data, partial- and

full-data cuts, data for multiple frequencies, and unidentified

data.

The file format provides for new fields in future OpenPF

revisions without the overhead of reserved bytes.  New fields

can be appended to any file block.  File parsers that use block

length to advance to the next block will skip over new fields.

OpenPF uses standard coordinate systems.  For

rectangular coordinates, X and Y are in the horizontal plane and

Z is height.  For spherical coordinates, R is radial distance,

phi is azimuth angle, and theta is zenith angle.  For

cylindrical coordinates, rho is horizontal distance, phi is

azimuth angle, and z is height.  Azimuth angle is 0 along the X

axis and increases in the XY plane toward the Y axis

(counterclockwise).  Zenith angle is 0 along the Z axis and

increases in the Z-phi plane toward the phi ray.  Units are

meters and degrees.

OpenPF files use the Intel-processor "little-endian"

data convention (least-significant byte first for multibyte

data).  Bit number 0 is the least-significant bit.  Bits marked

as reserved are undefined and should not be assumed to be 0.

OpenPF files use the file extension ".PF".

In the descriptions that follow, word means a 16-bit

unsigned integer and FP means a 32-bit, single-precision,

floating-point number that conforms to the IEEE-754-1985

standard.  ASCII means a character string that uses the

extended-ASCII PC-8 symbol set.

Header

OpenPF files begin with a block of the following format:

Version                 byte

Header length           word

Source length           byte

Title length            byte

Environment length      byte

Notes length            word

Source                  ASCII

Title                   ASCII

Environment             ASCII

Notes                   ASCII

Version

This byte identifies the OpenPF version used to generate

the file.  The version number consists of a major revision (an

integer) and a minor revision (an integer between 0 and 9)

separated by a decimal point.  The upper nibble (4 bits)

contains the major revision and the lower nibble the minor.

Header Length

This field is the total length of the header in bytes.

File parsers should use this length to advance past the header

instead of assuming that data blocks begin immediately after the

notes string.  Future OpenPF revisions may define additional

fields after the notes string.  Header lengths less than 3 are

invalid.

Source, Title, Environment, and Notes Lengths

These fields specify the lengths in bytes of the ASCII

strings.  Zero is valid.

Source String

This string identifies the data source.  This may be an

antenna-analysis program, an antenna-test range, a laboratory,

etc.  No bytes are allocated for a zero-length string.

Title, Environment, and Notes Strings

The title string describes the antenna.  The environment

string describes the antenna environment (free space, antenna

height, ground constants, etc.).  The notes string contains

auxiliary information that might be displayed only upon request.

Data blocks that follow may contain strings that override these

global strings.  No bytes are allocated for zero-length strings.

Data Blocks

Data following the header is organized into blocks.

Each data block begins with block type (a byte) followed by

block length (a word).  Block types 0 through 127 are standard

and reserved.  Programs may use types 128 through 255 for

special purposes, but block length must always follow block type

so that file parsers can skip over the block.  Blocks may appear

in any order.  The following standard blocks are defined:

NOP Block

Block type              byte (0)

Block length            word

Rest of block

Block type 0 is a no-operation block.  Its purpose is to

provide a standard ignore-me block.  Normally this block isn't

used, but it may be convenient when patching a file.  Block

length is the total length of the block in bytes.  Block lengths

less than 3 are invalid.

Relative Far-Field Blocks

Block type              byte (1-16)

Block length            word

Title length            byte

Environment length      byte

Notes length            word

Frequency               FP

Plane                   byte

Plane angle             FP

Symmetry                byte

Number of points        word

First angle             FP

Angular increment       FP

Data points             each FP

Title                   ASCII

Environment             ASCII

Notes                   ASCII

Block Type

Block types 1-16 contain planar cuts of relative far-

field data as follows:

Block           Data                    Units

1     Total magnitude                 dBi

2     Horizontal magnitude            dBi

3     Vertical magnitude              dBi

4     Right-circular magnitude        dBic

5     Left-circular magnitude         dBic

6     Major-axis magnitude            dBi

7     Minor-axis magnitude            dBi

8     Ellipticity                     dB

9     Total phase                     degrees

10     Horizontal phase                degrees

11     Vertical phase                  degrees

12     Right-circular phase            degrees

13     Left-circular phase             degrees

14     Major-axis phase                degrees

15     Minor-axis phase                degrees

16     Polarization tilt               degrees

Ellipticity is the length of the minor axis of the

polarization ellipse divided by the length of the major axis

expressed in dB (the IEEE-754-1985 floating-point code for

-infinity is valid).  Polarization tilt is the counterclockwise

angle between the major axis and the E(theta) direction.

Block Length

This field is the total length of the block in bytes.

Block lengths less than 3 are invalid.

Title, Environment, and Notes Lengths

These fields specify the lengths of the ASCII strings in

bytes.  Zero is valid.

Frequency

Frequency is in MHz.

Plane

This byte is 0 for azimuth data (constant zenith angle)

and 1 for elevation data (constant azimuth angle).  Other values

are reserved.

Plane Angle

This field is the zenith angle for azimuth data and the

azimuth angle for elevation data.

Symmetry

The bits of this byte have the following meaning when

set:

Bit     Azimuth Data       Elevation Data

0      X symmetry         XY symmetry

1      Y symmetry         Z symmetry

Symmetry means that the data has the same value when

reflected about the line or plane indicated.  For azimuth data,

bits 0 and 1 refer to the X and Y axes.  For elevation data,

they refer to the XY plane and the Z axis.  Both bits may be set

simultaneously.  Bits 2-7 are reserved.

Number of Points

This field is the number of data points that follow.  0

is valid.

First Angle

This field is the angle of the first data point.

Angular Increment

This field defines the sign and magnitude of the angular

difference between successive data points.

Data Points

The data set begins at the first angle.  The angles of

successive data samples differ by the angular increment.

Title, Environment, and Notes Strings

When string length is nonzero, the string overrides the

corresponding header string for this block.  When zero, the

header string applies for this block and no string space is

allocated.

Absolute Near- and Far-Field Blocks

�               Block type              byte (64-81 & 96-101)

Block length            word

Title length            byte

Environment length      byte

Notes length            word

Frequency               FP

Power                   FP

Coordinate system       byte

Symmetry                byte

Number of A points      word

First A                 FP

A increment             FP

Number of B points      word

First B                 FP

B increment             FP

Number of C points      word

First C                 FP

C increment             FP

Data points             each FP

Title                   ASCII

Environment             ASCII

Notes                   ASCII

Block Type

Block types 64-81 contain absolute fields.  Block types

67-81 are field components resolved in rectangular coordinates.

These blocks are defined as follows:

Block           Data            Units

64     Power density           watts/square-meter

65     Peak E magnitude        volts/meter

66     Peak H magnitude        amps/meter

67     Px Poynting vector      watts/square-meter

68     Py Poynting vector      watts/square-meter

69     Pz Poynting vector      watts/square-meter

70     Ex magnitude            volts/meter

71     Ey magnitude            volts/meter

72     Ez magnitude            volts/meter

73     Hx magnitude            amps/meter

74     Hy magnitude            amps/meter

75     Hz magnitude            amps/meter

76     Ex phase                degrees

77     Ey phase                degrees

78     Ez phase                degrees

79     Hx phase                degrees

80     Hy phase                degrees

81     Hz phase                degrees

Block types 96-101 contain absolute electric-field

components resolved in spherical coordinates as follows:

Block           Data            Units

96     E(R) magnitude          volts/meter

97     E(phi) magnitude        volts/meter

98     E(theta) magnitude      volts/meter

99     E(R) phase              degrees

100     E(phi) phase            degrees

101     E(theta) phase          degrees

Use block type 65 for peak E magnitude.

Block Length, String Lengths, and Frequency

These fields are the same as for relative far-field

blocks.

Power

This field is antenna input power in watts.

Coordinate System

This byte specifies the coordinate system used to locate

data points (it does not specify the coordinate system used to

resolve field components).  The byte is 0 for rectangular

coordinates, 1 for spherical coordinates, and 2 for cylindrical

coordinates.  Other values are reserved.  Coordinates are

defined as follows:

Coordinate System       A       B       C

0     Rectangular       X       Y       Z

1     Spherical         R       phi     theta

2     Cylindrical       rho     phi     z

Symmetry

The low-order bits of this byte have the following

meaning when set:

Bit     Meaning

0      A-coordinate symmetry

1      B-coordinate symmetry

2      C-coordinate symmetry

Here, symmetry means that the data has the same value

when the coordinate is negated (this definition is not the same

as that for relative fields).  Any of the bits may be set

simultaneously.  Bits 3-7 are reserved.

Number of A, B, or C points

These fields specify the number of points for each

coordinate.  0 is invalid.  The total number of data points is

the product of A, B, and C.  For rectangular coordinates, the

field is computed over a point, line, rectangular surface, or

rectangular volume.  For spherical coordinates, the field is

computed over a point, line, circle, disc, spherical surface,

spheroid, or portion thereof.  For cylindrical coordinates, the

field is computed over a point, line, circle, disc, cylindrical

surface, cylindrical volume, or portion thereof.

First A, B, and C

These fields specify the starting values of each

coordinate.

A, B, and C Increments

These fields define the sign and magnitude of the

difference between successive coordinate values.

Data Points

Points are ordered as follows:  A cycles from first to

last, then B, and finally C.  The coordinate values of

successive data samples differ by the A, B, or C increments.

Title, Environment, and Notes Strings

These fields are the same as for relative far-field

blocks.